# UNIMAG Interface – Power Supply EPICS IOC Specification

## Process Variables (PVs)

### Current Control

- Current Readback (CURRENT_RB)
Provides the measured output current of the magnet channel.
PV format:

- $(P):$(R):CURRENT_RB

- Current Setpoint (CURRENT_SP)
Sets the desired current for the magnet channel.
PV format:

- $(P):$(R):CURRENT_SP

### Channel State Control

- Channel State Readback (STATE_RB)
Reports the current operational state of the magnet channel.
PV format:

- $(P):$(R):STATE_RB

- Channel State Setpoint (STATE_SP)
Allows the user to request a change of the channel state (e.g. via ON, STANDBY, RESET commands).
PV format:

- $(P):$(R):STATE_SP

## Configuration Parameters

- set_tolerance
Allowed tolerance on CURRENT_RB when checking if the requested current has been reached.
Default: 1

- zero_tolerance
Allowed tolerance when checking that the current is zero.
Default: 1

- set_timeout_s
Timeout (in seconds) to verify that the requested current has been reached.
If the timeout expires, the operation is considered failed.
Default: 10

- max_retries
Number of retries before declaring STATE_NOT_REACHED.

## Channel State Values (STATE_RB, STATE_SP)

The channel state PVs represent the operational status of the magnet channel.

Possible values:

- OFF – Magnet is powered off

- ON – Magnet is powered and active

- STANDBY – Magnet is in standby mode

- FAULT – A fault condition has been detected

- RESET – Reset operation in progress

## UNIMAG Machine State

- UNIMAG State Readback (UNIMAG_STATE_RB)
Reports the internal UNIMAG state machine status.
PV format:

- $(P):$(R):UNIMAG_STATE_RB

### Possible Values

- OK
The power supply is ready to accept new commands.

- STATE_NOT_REACHED
The requested state or current was not reached within tolerance and timeout.
This is a low-level warning; the machine remains ready to accept new commands.

- ZERO_STBY
The magnet is ramping down to zero current and transitioning to standby.

- CHANGE_POL
A polarity change is in progress.

- GOING_TO_SET
The magnet is ramping toward the requested current setpoint.

## UNIMAG State Machine Behavior

### Current Setpoint Handling (CURRENT_SP)

When a new value is written to CURRENT_SP, the UNIMAG state machine operates as follows:

- Polarity Check

  - The controller checks whether a polarity change is required.

  - If a polarity change is needed, the state transitions to:

  - ZERO_STBY → CHANGE_POL

- Ramp to Zero (if required)

  - The current is ramped to zero.

  - Zero is considered reached when:

  - |CURRENT_RB| ≤ zero_tolerance

  - The ramp must complete within set_timeout_s.

  - If zero is not reached within the timeout (after max_retries attempts), the state becomes:

  - STATE_NOT_REACHED

- Set Current

  - If the current polarity already matches the desired polarity:

    - The system transitions directly to:

    - GOING_TO_SET

    - The current is ramped to the requested value.

    - Success condition:

    - |CURRENT_RB − CURRENT_SP| ≤ set_tolerance

    - If set_tolerance is 0, no verification is performed and the state transitions directly to OK.

- Polarity Mismatch

  - If the current polarity does not match the requested polarity:

    - Ramp to zero (ZERO_STBY)

    - Perform polarity change (CHANGE_POL)

    - Then proceed with step 3.

### Transition from ON to STANDBY

- If transitioning from ON to STANDBY:

  - If:

  - |CURRENT_RB| > zero_tolerance

the system must first ramp to zero (ZERO_STBY).

  - If the current is already within zero tolerance, the system can transition directly to STANDBY.

If you want, I can also:

- Convert this into formal EPICS documentation

- Define state diagrams

- Suggest PV naming refinements

- Align it with EPICS record types (bo, ao, mbbi, calc, etc.)

Just tell me 👍

can you add the retry logic on state transitions max_retries, can you suggest refinements?

Below is an extended and refined version of the specification that explicitly defines the retry logic (max_retries) for state transitions, clarifies failure semantics, and suggests architectural refinements that fit well with EPICS IOC design and operations practice.

# UNIMAG Interface – Power Supply EPICS IOC

State Machine, Retry Logic, and Refinements

## Retry Logic (max_retries)

max_retries defines how many times a state transition or verification step is retried before declaring a failure.

### General Rules

- Each verifiable transition (e.g. ramp-to-zero, ramp-to-set) is attempted up to:

- max_retries + 1

total times (initial attempt + retries).

- A retry is triggered when:

  - The expected condition is not met within set_timeout_s

  - Or the power supply reports an incompatible or invalid state

- Between retries:

  - The system returns to a safe intermediate state (ZERO_STBY or STANDBY)

  - No user command is required to trigger retries

- If all retries fail:

  - UNIMAG_STATE_RB is set to STATE_NOT_REACHED

  - STATE_RB remains unchanged

  - The IOC remains responsive to new commands

## Retry-Enabled State Transitions

### 1. Ramp to Zero (ZERO_STBY)

Entry Condition

- Polarity change required

- Transition from ON → STANDBY with |CURRENT_RB| > zero_tolerance

Success Condition

|CURRENT_RB| ≤ zero_tolerance

within set_timeout_s

Retry Behavior

- If timeout expires:

  - Increment retry counter

  - Re-issue zero-current command

  - Remain in ZERO_STBY

- After max_retries failures:

  - Transition to STATE_NOT_REACHED

  - Exit retry loop

### 2. Polarity Change (CHANGE_POL)

Entry Condition

- Successful completion of ZERO_STBY

- Requested polarity differs from current polarity

Success Condition

- Hardware reports correct polarity

Retry Behavior

- If polarity does not change within set_timeout_s:

  - Retry polarity change

- After max_retries failures:

  - Transition to STATE_NOT_REACHED

### 3. Ramp to Setpoint (GOING_TO_SET)

Entry Condition

- Correct polarity is established

Success Condition

|CURRENT_RB − CURRENT_SP| ≤ set_tolerance

within set_timeout_s
(or immediate success if set_tolerance == 0)

Retry Behavior

- On timeout:

  - Increment retry counter

  - Re-issue setpoint command

- After max_retries failures:

  - Transition to STATE_NOT_REACHED

## Retry Counter Handling

- Retry counters are:

  - Local to each transition

  - Reset upon successful completion of that transition

- Retry counters are also reset when:

  - A new CURRENT_SP is written

  - A new STATE_SP command is issued

## Failure Semantics (STATE_NOT_REACHED)

When STATE_NOT_REACHED is entered:

- The power supply remains in the last known safe electrical state

- UNIMAG_STATE_RB = STATE_NOT_REACHED

- The IOC:

  - Accepts new commands

  - Does not auto-retry unless a new command is issued

- A transition to RESET or a new CURRENT_SP clears the error

## Suggested Refinements

### 1. Explicit Intermediate State Tracking

Suggestion:
Expose the active sub-state via an internal or optional PV:

$(P):$(R):UNIMAG_STEP_RB

Possible values:

- CHECK_POL

- RAMP_TO_ZERO

- POLARITY_CHANGE

- RAMP_TO_SET

- VERIFY

This improves:

- Debugging

- Operator understanding

- Alarm correlation

### 2. Explicit Alarm Severity Mapping

Map failures to EPICS alarm severities:

| Condition | PV | Severity |
| --- | --- | --- |
| Timeout (retrying) | UNIMAG_STATE_RB | MINOR |
| STATE_NOT_REACHED | UNIMAG_STATE_RB | MAJOR |
| FAULT | STATE_RB | INVALID |

### 3. Add a Busy Indicator

Add:

$(P):$(R):BUSY_RB

- 1 → State machine active

- 0 → Idle / ready

Useful for:

- GUIs

- Script synchronization

- Preventing overlapping commands

### 4. Command Arbitration Rules

Refinement:

- If a new CURRENT_SP arrives while BUSY_RB = 1:

  - Abort current sequence

  - Reset retry counters

  - Restart state machine from polarity check

This avoids deadlocks and race conditions.

### 5. Optional Retry Backoff

Add (optional):

retry_delay

- Delay (seconds) between retries

- Prevents hammering slow or overloaded power supplies

### 6. Formal Transition Table (Optional)

If you want to formalize further, define transitions as:

| From | Trigger | To | Verify | Retry |
| --- | --- | --- | --- | --- |
| ON | CURRENT_SP | GOING_TO_SET | ΔI ≤ err | yes |
| ON | POL mismatch | ZERO_STBY | I≈0 | yes |
| ZERO_STBY | Zero reached | CHANGE_POL | pol ok | yes |
| CHANGE_POL | Pol OK | GOING_TO_SET | ΔI ≤ err | yes |